# DA (Door Access) Encoder Procedures

## Log In

- To log in, swipe the Res Life log in card with the mag stripe facing out / away from you.

NOTE: This is NOT the user ID card to be updated

- After Success message, Enter PIN code and click OK

## Check for DA Sector Encoding

- After a successful login, two tiles will be displayed:

- Select the Card View tile to view the current user ID card details

- At the Card View screen, select the Full Read Mode button

- Place card on the Tap or Swipe Credential circle and hold it there

- The Full Read Report screen is displayed

- Check for the DA sector encoding. If you only see a Transact Application and a Third Party Application row, the card needs to be updated.

- Select X or the back arrow to return to the main screen (the screen with the two tiles)

## Encode Card

The Encode Card option allows you to encode a Door Access (DA) sector by copying data from a swiped card. The Door Access sector information is required for automatic / lobby doors to work.

- Select X in the upper-right of the screen until the main Transact screen is displayed

- Select the Encode Card tile

- The Encode Card screen displays

- Swipe the ID card to be updated with the mag stripe facing away from you

- Place card on the Hold Card Here circle and hold it there

NOTE: you may have to readjust the placement of the card slightly

- The Success screen displays briefly

## Verify Door Access (DA) Encoding

- Select X or the back arrow to return to the main screen

- Following the procedures to perform a Full Read on the card

- At the Full Read Report screen, you should now see a Transact Application row, a Third Party Application row, and a Door Access Application row.